SkyKin / Ahununu Call Center

Inbound Performance Report

Period: Last 7 days (as of 25 Sep 2026 ~08:40 UTC / 11:40 EAT)
Source: FreeSWITCH CDR (v_xml_cdr) on ecs-cc + FreeSWITCH logs (sample cases)
Scope: Inbound DIDs matching 11619803*

# 1. Executive summary

| Finding | Result |
| --- | --- |
| Platform health | Healthy (FreeSWITCH, DB, web up ~10 days; WS-SIP active) |
| Inbound answer-to-agent rate | ~42% |
| Abandoned (system answered, no agent connected) | ~58% |
| Main driver of client complaints | Abandoned / not connected to a live agent |
| PBX systematically blocking agent Answer? | No evidence for ring-then-abandon set (~97% NORMAL_CLEARING) |
| Ownership | Mostly coverage + agent pickup; platform up; rare softphone/media issues |

# 2. Platform status (ops check)

| Component | Status |
| --- | --- |
| skykin-freeswitch | Up 10 days (healthy) |
| skykin-db | Up 10 days (healthy) |
| skykin-web | Up 10 days |
| skykin-ws-sip | active |
| Host load | ~0.20 (low) |
| Disk / | 74% used |
| Disk /var | 79% used (monitor recordings) |

Verdict: No platform outage at time of check.

# 3. Inbound KPI (7 days, reportable legs)

| Metric | Count | % |
| --- | --- | --- |
| Reportable inbound calls | 751 | 100% |
| Answered — agent connected | 315 | 41.9% |
| Abandoned — IVR/system answered, agent NOT connected | 436 | 58.1% |
| Missed (no IVR talk) | 0 | 0% |

Answered by agent = real bridge (cc_agent_bridged / bridge_uuid / wait+talk proof).

Abandoned = customer heard system (billsec > 0) but never connected to an agent.

# 4. Abandoned timing (EAT) — coverage vs open hours

| Window | Abandoned | Share of abandons |
| --- | --- | --- |
| Before 08:00 | 226 | ~52% |
| 08:00–18:00 | 209 | ~48% |
| After 18:00 | 1 | ~0% |
| Total abandoned | 436 | 100% |

Interpretation:

• ~Half of abandons are before shift / early morning → agents not logged in yet.

• ~Half are during daytime → Available/registered + answer discipline still required.

Hourly abandon peaks (EAT): 08:00 (55), 07:00 (44), 05:00 (42), 02:00 (40), 10:00 (40).

# 5. Abandoned: was an agent rung?

| Category | Count | Share |
| --- | --- | --- |
| Abandoned total (billsec > 0, not connected) | ~432 | 100% |
| Agent was offered/rung then abandoned | 170 | ~39% |
| No agent offered in CDR (remainder) | ~262 | ~61% |

## 5a. Ring-then-abandon by extension

| Agent ext | Abandoned after ring |
| --- | --- |
| 201 | 78 |
| 203 | 48 |
| 202 | 29 |
| 205 | 10 |
| 204 | 5 |

## 5b. Proof: system block vs no pickup (rung → abandoned)

| Hangup cause | Count | Meaning |
| --- | --- | --- |
| NORMAL_CLEARING | 162 | Caller/wait cleared; agent did not connect — not PBX block-answer |
| MEDIA_TIMEOUT | 2 | Possible media issue |
| NORMAL_UNSPECIFIED | 2 | Unclear |
| INTERWORKING | 1 | Network interworking |
| ALLOTTED_TIMEOUT / TEMPORARY_FAILURE / INCOMPATIBLE | 0 | None in this set |

Conclusion: For calls where an agent was rung and the call still abandoned, there is no CDR evidence that the PBX prevented Answer. Almost all cleared as normal without agent connect (= no pickup / caller gave up).

# 6. Softphone / media issues (separate, smaller)

| Cause (7 days sample) | Count |
| --- | --- |
| NORMAL_TEMPORARY_FAILURE | 46 |
| INCOMPATIBLE_DESTINATION | 4 |

Known verified cases (logs):

• Inbound silent after answer (e.g. 251938666888 ~14:46 EAT): agent bridged + DTLS OK; customer on recording; agent softphone silent → WebRTC/network.

• Outbound INCOMPATIBLE_DESTINATION (203 → 911502296): no suitable ICE candidates → agent WebRTC, number OK.

These are not the bulk of the ~58% abandon rate.

# 7. Mapping client complaint language

| Client statement | Finding |
| --- | --- |
| Calls not properly responded to | Confirmed — ~58% abandoned without agent connect |
| Call personal phones due to CC issues | Consistent with low connect rate |
| Appear answered in system but not connected to agent | Confirmed — PBX answers IVR/music first; if no agent connects → Abandoned |
| Permanent PBX outage | Not supported — platform healthy |

# 8. Recommendations

## Client / operations

1. Publish official open hours; after-hours announcement + callback.

2. All agents Registered + Available by shift start (e.g. 08:00).

3. Enforce answer when softphone rings (focus ext 201, then 203).

4. Report every silent-after-answer with time + caller + agent ext.

## SkyKin / platform

1. Keep monitoring platform health and disk (/var recordings).

2. Continue softphone/WebRTC hardening for rare media/ICE cases.

3. Weekly report: answered % / abandoned % / ring-no-answer by extension.

# 9. Bottom line

• Server: OK.

• Service: ~42% of inbound reach an agent; ~58% abandon after system answer.

• Complaints: Valid on unanswered / not connected to agent.

• Not mainly: PBX preventing agents from answering.

• Mainly: before-login coverage + daytime ring-no-answer / availability.

Report generated from ecs-cc CDR queries and FreeSWITCH log samples.